Example of limitations to reduce FPGA footprint:

- LOAD/STORE always fetches 32 bit. 8/16 bit access is emulated through 32 bit LOAD/STORE. Note that 8/16 bit may need to be atomic operations in the users programming model and hence they may be implemented in exception handlers.

- Although the users programming model will not be able to handle adding wait states during DRAM refresh/cache line fetch, wait states for memory access

- IOLOAD/STORE could be limited to a fixed width

Example:

If a LOAD operation accesses memory mapped IO(i.e. memory access outside DRAM range), an exception is caused which emulates the LOAD operation:

ioprefecth  ; charge up this data

wait:

iobusy

branch wait  ; is the CPU still waiting for the result?

ioload		; the data is available

Alternate base instructions

Although a stack based CPU is an interesting approach to reducing the complexity of a CPU, it is not the only alternative. A register based alternative might be prefereable in that it could be easier to write a GCC compiler backend.

Starting with a register based scheme, an instruction set based upon registers is investigated below.